# BLACK AND WHITE PYRANOMETER

## Model 8-48

The Black & White Pyranometer has a detector consisting of a differential thermopile with the hot-junction receivers blackened and the cold-junction receivers whitened. The receiver is of radial wire-wound plated construction with the black segments coated with a flat black coating and the white with Barium Sulfate. Built-in temperature compensation with thermistor circuitry is incorporated to free the instrument from effects of ambient temperature. A precision ground optical glass hemisphere of Schott glass WG295 uniformly transmits energy from 0.285 to 2.8 µm.

The cast aluminum case carries a circular spirit level and adjustable leveling screws. Also supplied is a desiccator, which can be readily inspected.

A calibration certificate traceable to the World Radiation Reference is included.

SPECIFICATIONS

- Sensitivity: approx. 10 µV/Wm-2.

- Impedance: approx. 350 Ohms.

- Temperature Dependence: ±1.5% over ambient temperature range -20 to +40°C.

- Linearity: ±1% from 0 to 1400 Wm-2.

- Response time: 5 seconds (1/e signal).

- Cosine:

  - ± 2 % from normalization 0-70° zenith angle;

  - ± 5 % 70-80° zenith angle.

- Mechanical Vibration: tested up to 20 g's without damage.

- Calibration: integrating hemisphere.

- Size: 5.75 inch diameter, 2.75 inches high.

- Weight: 2 pounds.